# IoT Assignment 3 — Solutions (Q36–Q50)

  - Student Name - Unnati Gupta

  - Enrollment/Roll Number - IOT13

  - Course Name: Internet of Things (IoT)

  - Assignment Title - Assignment 3

  - Submission Date - 7/7/2026

## Section C: Protocol Theory

### Q36. MQTT vs HTTP Comparison

| Parameter | MQTT | HTTP |
| --- | --- | --- |
| Architecture pattern | Publish/Subscribe via a central broker. Devices don't talk to each other directly; they publish to topics and subscribe to topics of interest. | Request/Response, client-server. Client must know the server's address and initiate every exchange. |
| Data transfer model | Persistent TCP connection; small binary/text payloads pushed as events occur. Broker handles routing and fan-out to many subscribers. | Each exchange typically opens a new connection (or uses keep-alive); payload includes verbose headers (HTTP headers, JSON body). |
| Power consumption | Low — the connection stays open with lightweight keep-alive pings (PINGREQ/PINGRESP), so radio wake-ups are minimized. Well suited to battery-powered nodes. | Higher — repeated TCP/TLS handshakes and larger headers mean more radio-on time per message. |
| Latency | Low latency — broker pushes data to subscribers instantly on publish. | Higher latency if polling is used (client must ask "any update?" repeatedly); can be low only if the server pushes via long-polling/webhooks, which adds complexity. |
| Use case suitability for IoT | Excellent — designed for constrained devices, unreliable networks, and many-to-many communication (e.g., 1000 sensors → 1 dashboard). | Better suited to occasional, one-off requests (e.g., REST API calls, firmware OTA downloads) rather than continuous streaming telemetry. |
| Security considerations | Supports TLS, and username/password or client-certificate authentication at the broker; ACLs can restrict which topics a device may publish/subscribe to. Public brokers without auth are a major risk. | Supports HTTPS/TLS, standard web auth (API keys, OAuth, JWT). Very mature ecosystem of security tooling, but larger attack surface (headers, cookies, more verbose protocol). |

Recommendation for a 1000-node smart agriculture deployment: MQTT. At that scale, thousands of low-power sensor nodes (soil moisture, temperature, humidity) need to publish small readings frequently while conserving battery. MQTT's publish/subscribe model lets a single broker fan data out to multiple consumers (a dashboard, a logging service, an alerting service) without each node needing to know about all of them. Its low bandwidth and power footprint over a persistent connection scales far better than 1000 nodes each repeatedly opening HTTP connections, and QoS levels let us tune reliability per sensor type (e.g., QoS 1 for irrigation commands, QoS 0 for routine telemetry).

### Q37. Wi-Fi Security for IoT

### IoT devices are frequent targets because they are often deployed with weak default credentials and rarely receive security patches. The most common Wi-Fi attack vectors include exploitation of WPA2 vulnerabilities such as KRACK (Key Reinstallation Attack), which forces reuse of encryption nonces during the 4-way handshake, allowing an attacker to decrypt traffic. Another major vector is the evil twin attack, where an attacker sets up a rogue access point broadcasting the same SSID as a trusted network; unsuspecting ESP32 devices configured to auto-connect by SSID name can associate with the attacker's AP instead, exposing credentials and data to interception.

To secure ESP32 Wi-Fi connections, best practice starts with migrating to WPA3, which uses Simultaneous Authentication of Equals (SAE) to resist offline dictionary attacks and provides forward secrecy, so a compromised password doesn't expose past traffic. For device-to-server communication, certificate pinning ensures the ESP32 only trusts a specific server certificate (or its public key), preventing man-in-the-middle attacks even if a malicious CA certificate is installed on the network. Devices should also disable auto-connect to open or unknown networks and validate the AP's BSSID, not just the SSID.

Hardcoding Wi-Fi passwords and API keys directly in source code is dangerous because that code is frequently pushed to public GitHub repositories, embedded in shared .ino files, or extractable from flash memory via UART/JTAG debugging. Anyone with the binary or repository access gains instant network credentials. Storing secrets in a separate, git-ignored config.h file, or better, in encrypted NVS (Non-Volatile Storage) partitions, keeps secrets out of version control and harder to extract.

References: (1) Vanhoef, M. & Piessens, F., "Key Reinstallation Attacks: Forcing Nonce Reuse in WPA2," ACM CCS 2017. (2) Wi-Fi Alliance, "WPA3 Specification," wi-fi.org.

### Q38. LoRa & LPWAN — Long Range IoT

LPWAN (Low Power Wide Area Network) is a class of wireless networking technology designed to connect devices over distances of several kilometers while consuming very little power, at the cost of low data throughput — a trade-off ideal for sensors that send small readings infrequently (temperature, soil moisture, meter readings) rather than streaming media.

LoRa (Long Range) achieves this reach at low power through Chirp Spread Spectrum (CSS) modulation combined with a configurable Spreading Factor (SF). A higher spreading factor "stretches" each bit across a longer chirp signal, making it recoverable even far below the noise floor, at the cost of a lower data rate and longer airtime. SF7 gives faster, shorter-range transmission, while SF12 sacrifices speed for maximum range (10–15 km in open terrain) and superior penetration through obstacles.

Compared to Zigbee (short range, ~10–100 m, mesh topology, higher data rate, used for home automation) and Z-Wave (similar short range, proprietary, used mainly in smart home locks/lighting), LoRa trades data rate for dramatically longer range and lower power, making it the better fit for outdoor, sparse-node deployments.

Real-world example: Karnataka's "Krishi Doot" and similar LoRaWAN-based smart agriculture pilots in Indian states use LoRa sensor nodes across farmland to monitor soil moisture and automate irrigation over multi-kilometer ranges without cellular connectivity.

### Q39. IoT Gateway

An IoT gateway is a hardware/software bridge that sits between resource-constrained sensor nodes and the internet/cloud. It aggregates data from multiple sensor nodes (which often use short-range, low-power protocols the internet doesn't understand directly), performs protocol translation, local buffering, and sometimes edge processing/filtering, before forwarding data onward using internet-compatible protocols.

Data flow diagram:

[IoT Sensor Nodes]  --(BLE / Zigbee / LoRa)-->  [Gateway]  --(Wi-Fi / Ethernet / Cellular)-->  [Cloud]  --(HTTPS / WebSocket)-->  [User Application]

(e.g., ESP32 +          Local protocol         (e.g., Raspberry Pi         MQTT/HTTPS           (AWS IoT Core,          Dashboard / Mobile App

DHT11, MQ-2)            translation            or ESP32 as edge          to broker/API         Firebase, ThingSpeak)   (Grafana, custom app)

hub)

Typical protocols at each layer:

- Sensor Node → Gateway: BLE, Zigbee, LoRa, or simple UART/I2C/analog for very short-range wired sensor-to-microcontroller links.

- Gateway → Cloud: MQTT (over TLS) is most common for telemetry; HTTPS/REST for occasional API calls or firmware updates.

- Cloud → User Application: HTTPS REST APIs for on-demand data, WebSockets or MQTT-over-WebSocket for real-time dashboard updates.

### Q40. MQTT QoS Levels

- QoS 0 (At most once): "Fire and forget." The message is sent once with no acknowledgment or retry. It may be lost if the network drops it, but there is minimal overhead.

- QoS 1 (At least once): The sender keeps the message until it receives a PUBACK from the receiver. If the ACK is lost, the message is resent, so the message is guaranteed to arrive — but it might arrive more than once (duplicates possible).

- QoS 2 (Exactly once): Uses a 4-step handshake (PUBREC, PUBREL, PUBCOMP) to guarantee the message is delivered exactly one time, no duplicates, no loss — but with the highest latency and overhead of the three.

For an IoT water level monitoring system: I would choose QoS 1. Losing a single water-level reading (as QoS 0 might) is risky in a flood-warning context — a missed CRITICAL reading could have real consequences. However, QoS 2's extra handshake latency isn't worth it here, because occasional duplicate readings are harmless (the next reading a few seconds later will simply confirm or update the water level) — the application can tolerate duplicates but not silent loss. QoS 1 gives the best balance of reliability and speed for this safety-relevant but duplicate-tolerant use case.

## Module 5: Advanced IoT Projects & Problem Statements

Note: each project below outlines the design approach and provides representative Arduino/ESP32 (C++) code implementing the core logic described in the requirements. For submission, this code should be placed in its own GitHub repository with a README (problem statement, approach, hardware list, circuit diagram), 5+ meaningful commits, and a short demo video, as required by the assignment guidelines.

### Q41. Smart Home Air Quality Monitor

Approach: The MQ-2 gas sensor gives an analog reading proportional to smoke/CO concentration. We read this alongside DHT11 temperature/humidity, classify the reading into SAFE / MODERATE / DANGER bands using two thresholds, drive an RGB LED and a buzzer with distinct tones per level, and log everything to Serial in CSV format so it survives without internet connectivity (fully local logic — no cloud dependency for core function).

cpp

- #include <DHT.h>

- #define MQ2_PIN     34      // analog input

- #define DHT_PIN     4

- #define DHT_TYPE    DHT11

- #define LED_RED     25

- #define LED_GREEN   26

- #define LED_BLUE    27

- #define BUZZER_PIN  14

- // Thresholds (analog units, calibrate against clean air baseline)

- const int MODERATE_THRESHOLD = 350;

- const int DANGER_THRESHOLD   = 600;

- DHT dht(DHT_PIN, DHT_TYPE);

- unsigned long lastRead = 0;

- const unsigned long INTERVAL = 1000; // 1 second

- void setup() {

- Serial.begin(115200);

- pinMode(LED_RED, OUTPUT);

- pinMode(LED_GREEN, OUTPUT);

- pinMode(LED_BLUE, OUTPUT);

- pinMode(BUZZER_PIN, OUTPUT);

- dht.begin();

- Serial.println("Timestamp,MQ-2 Value,DHT11 Temp,DHT11 Humidity,Severity Level");

- }

- void setRGB(bool r, bool g, bool b) {

- digitalWrite(LED_RED, r);

- digitalWrite(LED_GREEN, g);

- digitalWrite(LED_BLUE, b);

- }

- void soundAlarm(String level) {

- if (level == "SAFE") {

- noTone(BUZZER_PIN);

- } else if (level == "MODERATE") {

- tone(BUZZER_PIN, 1000, 200);

- } else {

- tone(BUZZER_PIN, 2500, 500);

- }

- }

- String formatTimestamp() {

- unsigned long totalSec = millis() / 1000;

- int mm = (totalSec / 60) % 60;

- int ss = totalSec % 60;

- char buf[6];

- sprintf(buf, "%02d:%02d", mm, ss);

- return String(buf);

- }

- void loop() {

- if (millis() - lastRead >= INTERVAL) {

- lastRead = millis();

- int gasValue = analogRead(MQ2_PIN);

- float temp = dht.readTemperature();

- float hum  = dht.readHumidity();

- String severity;

- if (gasValue >= DANGER_THRESHOLD) {

- severity = "DANGER";

- setRGB(true, false, false);

- } else if (gasValue >= MODERATE_THRESHOLD) {

- severity = "MODERATE";

- setRGB(false, false, true);

- digitalWrite(LED_GREEN, HIGH);

- } else {

- severity = "SAFE";

- setRGB(false, true, false);

- }

- soundAlarm(severity);

- Serial.print(formatTimestamp()); Serial.print(",");

- Serial.print(gasValue); Serial.print(" ppm,");

- Serial.print(temp); Serial.print("C,");

- Serial.print(hum); Serial.print("%,");

- Serial.println(severity);

- // Bonus: Bluetooth alert (via HC-05/HC-06 on Serial2 on ESP32)

- // Serial2.println(severity + ": Gas=" + String(gasValue));

- }

}

Design notes: Thresholds should be calibrated against a clean-air baseline taken during setup (MQ-2 needs a warm-up period of ~24–48 hours for stable readings; for a class demo, a shorter warm-up with documented caveats is acceptable). The system requires no Wi-Fi/cloud connection for its core safety function — everything runs locally on the ESP32, satisfying the "must work without internet" requirement.

### Q42. Automated Plant Watering System

Approach: Read soil moisture on a timed cycle, classify into dry/optimal/wet zones, check an LDR-based "rain proxy" before allowing irrigation, support a manual override button with a lockout period, and display status on a 16x2 LCD using non-blocking millis() timing.

cpp

- #include <LiquidCrystal.h>

- #define SOIL_PIN     34

- #define LDR_PIN      35

- #define RELAY_PIN    26

- #define OVERRIDE_BTN 27

- LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

- const unsigned long READ_INTERVAL    = 30UL * 60UL * 1000UL; // 30 minutes

- const unsigned long OVERRIDE_LOCKOUT = 5UL * 60UL * 1000UL;  // 5 minutes

- const int DRY_THRESHOLD      = 300;

- const int WET_THRESHOLD      = 700;

- const int RAIN_LDR_THRESHOLD = 200;

- unsigned long lastCheck = 0;

- unsigned long overrideStartTime = 0;

- unsigned long lastWateredTime = 0;

- bool overrideActive = false;

- void setup() {

- Serial.begin(115200);

- lcd.begin(16, 2);

- pinMode(RELAY_PIN, OUTPUT);

- pinMode(OVERRIDE_BTN, INPUT_PULLUP);

- digitalWrite(RELAY_PIN, LOW);

- }

- String moistureZone(int val) {

- if (val < DRY_THRESHOLD) return "DRY";

- if (val > WET_THRESHOLD) return "WET";

- return "OPTIMAL";

- }

- void loop() {

- unsigned long now = millis();

- if (digitalRead(OVERRIDE_BTN) == LOW && !overrideActive) {

- overrideActive = true;

- overrideStartTime = now;

- digitalWrite(RELAY_PIN, HIGH);

- lastWateredTime = now;

- }

- if (overrideActive && (now - overrideStartTime >= OVERRIDE_LOCKOUT)) {

- overrideActive = false;

- digitalWrite(RELAY_PIN, LOW);

- }

- if (!overrideActive && (now - lastCheck >= READ_INTERVAL)) {

- lastCheck = now;

- int soilVal = analogRead(SOIL_PIN);

- int ldrVal  = analogRead(LDR_PIN);

- String zone = moistureZone(soilVal);

- bool rainDetected = (ldrVal < RAIN_LDR_THRESHOLD);

- if (zone == "DRY" && !rainDetected) {

- digitalWrite(RELAY_PIN, HIGH);

- lastWateredTime = now;

- delay(3000); // brief watering burst

- digitalWrite(RELAY_PIN, LOW);

- }

- Serial.print("Soil="); Serial.print(soilVal);

- Serial.print(" Zone="); Serial.print(zone);

- Serial.print(" Rain="); Serial.println(rainDetected ? "YES" : "NO");

- // ESP8266 Wi-Fi logging to Google Sheets via IFTTT webhook (pseudo-call)

- // sendToGoogleSheets(soilVal, zone, rainDetected);

- }

- lcd.setCursor(0, 0);

- lcd.print("Mode:");

- lcd.print(overrideActive ? "MANUAL " : "AUTO   ");

- lcd.setCursor(0, 1);

- lcd.print("Last:");

- lcd.print((now - lastWateredTime) / 1000);

- lcd.print("s ago ");

}

Design notes: The rain proxy check (!rainDetected) prevents overwatering saffron during overcast periods, per the problem statement. The Wi-Fi logging function is stubbed as a comment since it needs network credentials in a separate config.h (see Q37 on why secrets shouldn't be hardcoded) — the real submission would call an IFTTT Maker Webhook or Google Apps Script endpoint over HTTPS.

### Q43. Smart Door Lock with OTP

Approach: Capture a 4-digit ID via keypad, generate a random 6-digit OTP, transmit it over Bluetooth (HC-05), and require the correct OTP within a non-blocking 30-second window before actuating a servo to "unlock."

cpp

- #include <Keypad.h>

- #include <LiquidCrystal.h>

- #include <Servo.h>

- LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

- Servo doorServo;

- const byte ROWS = 4, COLS = 4;

- char keys[ROWS][COLS] = {

- {'1','2','3','A'},

- {'4','5','6','B'},

- {'7','8','9','C'},

- {'*','0','#','D'}

- };

- byte rowPins[ROWS] = {9, 8, 7, 6};

- byte colPins[COLS] = {5, 4, 3, 2}; // adjust to avoid LCD pin conflicts in real wiring

- Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

- String enteredID = "";

- String enteredOTP = "";

- String generatedOTP = "";

- bool waitingForOTP = false;

- unsigned long otpStartTime = 0;

- const unsigned long OTP_TIMEOUT = 30000;

- void setup() {

- Serial.begin(115200); // HC-05 wired to hardware Serial or SoftwareSerial

- lcd.begin(16, 2);

- doorServo.attach(13);

- doorServo.write(0); // locked position

- randomSeed(analogRead(A0));

- lcd.print("Enter 4-digit ID");

- }

- String generateOTP() {

- int otp = random(100000, 999999);

- return String(otp);

- }

- void resetState() {

- delay(1500);

- enteredID = "";

- enteredOTP = "";

- waitingForOTP = false;

- lcd.clear();

- lcd.print("Enter 4-digit ID");

- }

- void loop() {

- char key = keypad.getKey();

- if (!waitingForOTP) {

- if (key) {

- if (key == '#') {

- if (enteredID.length() == 4) {

- generatedOTP = generateOTP();

- Serial.println("OTP:" + generatedOTP); // sent over Bluetooth to phone

- waitingForOTP = true;

- otpStartTime = millis();

- enteredOTP = "";

- lcd.clear();

- lcd.print("Enter OTP:");

- } else {

- lcd.clear();

- lcd.print("Invalid ID len");

- enteredID = "";

- }

- } else if (key >= '0' && key <= '9') {

- enteredID += key;

- lcd.setCursor(0, 1);

- lcd.print(String(enteredID.length()) + " digits*");

- }

- }

- } else {

- unsigned long elapsed = millis() - otpStartTime;

- if (elapsed >= OTP_TIMEOUT) {

- lcd.clear();

- lcd.print("OTP Expired");

- Serial.println("LOG: ID=" + enteredID + " OTP_TIMEOUT");

- resetState();

- return;

- }

- if (key) {

- if (key == '#') {

- if (enteredOTP == generatedOTP) {

- lcd.clear();

- lcd.print("Access Granted");

- doorServo.write(90); // unlock

- Serial.println("LOG: ID=" + enteredID + " ACCESS_GRANTED");

- delay(3000);

- doorServo.write(0);  // re-lock

- } else {

- lcd.clear();

- lcd.print("Access Denied");

- Serial.println("LOG: ID=" + enteredID + " ACCESS_DENIED");

- }

- resetState();

- } else if (key >= '0' && key <= '9') {

- enteredOTP += key;

- lcd.setCursor(0, 1);

- String masked = "";

- for (unsigned int i = 0; i < enteredOTP.length(); i++) masked += "*";

- lcd.print(masked);

- }

- }

- }

}

Design notes: The countdown uses millis() rather than delay(), so the keypad stays responsive throughout the 30-second window as required. All access attempts (granted/denied/timeout) are logged to Serial for auditing.

### Q44. PIR-Based Security Camera Trigger

Approach: Read PIR motion state, gate alerts by a configurable time window (set via Serial command), escalate through three alarm stages, flash an LED sequence, and (optionally) notify via a Telegram bot over Wi-Fi.

cpp

- #define PIR_PIN      27

- #define POT_PIN      34   // sensitivity potentiometer

- #define BUZZER_PIN   14

- #define LED1 25

- #define LED2 26

- #define LED3 33

- int startHour = 22, endHour = 6; // default night-time window

- unsigned long motionStart = 0;

- bool motionActive = false;

- int stage = 0; // 0=none, 1=warning, 2=alarm, 3=urgent

- int currentHour = 23; // replace with RTC module or NTP time in production

- void setup() {

- Serial.begin(115200);

- pinMode(PIR_PIN, INPUT);

- pinMode(BUZZER_PIN, OUTPUT);

- pinMode(LED1, OUTPUT); pinMode(LED2, OUTPUT); pinMode(LED3, OUTPUT);

- Serial.println("Ready. Send: SET_HOURS <start> <end>");

- }

- bool withinWindow() {

- if (startHour < endHour) {

- return currentHour >= startHour && currentHour < endHour;

- } else {

- return currentHour >= startHour || currentHour < endHour;

- }

- }

- void handleSerialCommand() {

- if (Serial.available()) {

- String cmd = Serial.readStringUntil('\n');

- cmd.trim();

- if (cmd.startsWith("SET_HOURS")) {

- int sh, eh;

- sscanf(cmd.c_str(), "SET_HOURS %d %d", &sh, &eh);

- startHour = sh; endHour = eh;

- Serial.println("Window updated: " + String(startHour) + " - " + String(endHour));

- }

- }

- }

- void flashSequence() {

- digitalWrite(LED1, HIGH); delay(100); digitalWrite(LED1, LOW);

- digitalWrite(LED2, HIGH); delay(100); digitalWrite(LED2, LOW);

- digitalWrite(LED3, HIGH); delay(100); digitalWrite(LED3, LOW);

- }

- void loop() {

- handleSerialCommand();

- bool motionDetected = digitalRead(PIR_PIN) == HIGH;

- if (motionDetected && withinWindow()) {

- if (!motionActive) {

- motionActive = true;

- motionStart = millis();

- stage = 1;

- Serial.println("MOTION @ " + String(millis()) + "ms - WARNING stage");

- }

- unsigned long elapsed = millis() - motionStart;

- if (elapsed > 10000 && stage < 3) {

- stage = 3;

- Serial.println("ESCALATED - URGENT stage");

- } else if (elapsed > 4000 && stage < 2) {

- stage = 2;

- Serial.println("ESCALATED - ALARM stage");

- }

- flashSequence();

- if (stage == 1) tone(BUZZER_PIN, 800, 150);

- else if (stage == 2) tone(BUZZER_PIN, 1500, 150);

- else tone(BUZZER_PIN, 3000, 300);

- // ESP32 Telegram bot notification (requires Wi-Fi + bot token in config.h)

- // sendTelegramAlert("Motion detected! Stage: " + String(stage));

- } else if (!motionDetected) {

- motionActive = false;

- stage = 0;

- noTone(BUZZER_PIN);

- }

}

Design notes: withinWindow() handles a night window that wraps past midnight (e.g., 22:00–06:00). In a real deployment, currentHour should come from an RTC module (e.g., DS3231) or NTP sync over Wi-Fi rather than being hardcoded.

### Q45. Smart Greenhouse Controller

Approach: Monitor temperature/humidity (DHT11) and light (LDR with running average), and drive three relays (heater, fan, grow light) using bang-bang control with hysteresis to avoid rapid relay chattering at the boundary.

cpp

- #include <DHT.h>

- #include <LiquidCrystal.h>

- #define DHT_PIN 4

- #define DHT_TYPE DHT11

- #define LDR_PIN 34

- #define HEATER_RELAY 25

- #define FAN_RELAY    26

- #define LIGHT_RELAY  27

- DHT dht(DHT_PIN, DHT_TYPE);

- LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

- const float TEMP_LOW = 18.0, TEMP_HIGH = 22.0, HYSTERESIS = 1.0;

- const float HUM_HIGH = 70.0;

- const int SAMPLES = 10;

- int ldrReadings[SAMPLES];

- int sampleIndex = 0;

- long ldrRunningTotal = 0;

- unsigned long lastCycle = 0;

- int displayPage = 0;

- void setup() {

- Serial.begin(115200);

- dht.begin();

- lcd.begin(16, 2);

- pinMode(HEATER_RELAY, OUTPUT);

- pinMode(FAN_RELAY, OUTPUT);

- pinMode(LIGHT_RELAY, OUTPUT);

- for (int i = 0; i < SAMPLES; i++) ldrReadings[i] = 0;

- }

- int updateLDRAverage() {

- ldrRunningTotal -= ldrReadings[sampleIndex];

- ldrReadings[sampleIndex] = analogRead(LDR_PIN);

- ldrRunningTotal += ldrReadings[sampleIndex];

- sampleIndex = (sampleIndex + 1) % SAMPLES;

- return ldrRunningTotal / SAMPLES;

- }

- void controlClimate(float temp, float hum) {

- static bool heaterOn = false;

- if (temp < TEMP_LOW - HYSTERESIS) heaterOn = true;

- else if (temp > TEMP_LOW + HYSTERESIS) heaterOn = false;

- digitalWrite(HEATER_RELAY, heaterOn ? HIGH : LOW);

- static bool fanOn = false;

- if (temp > TEMP_HIGH + HYSTERESIS || hum > HUM_HIGH + HYSTERESIS) fanOn = true;

- else if (temp < TEMP_HIGH - HYSTERESIS && hum < HUM_HIGH - HYSTERESIS) fanOn = false;

- digitalWrite(FAN_RELAY, fanOn ? HIGH : LOW);

- }

- void controlLight(int avgLdr) {

- static bool lightOn = false;

- const int LIGHT_THRESHOLD = 400;

- if (avgLdr < LIGHT_THRESHOLD - 30) lightOn = true;

- else if (avgLdr > LIGHT_THRESHOLD + 30) lightOn = false;

- digitalWrite(LIGHT_RELAY, lightOn ? HIGH : LOW);

- }

- void loop() {

- float temp = dht.readTemperature();

- float hum  = dht.readHumidity();

- int avgLdr = updateLDRAverage();

- controlClimate(temp, hum);

- controlLight(avgLdr);

- if (millis() - lastCycle >= 3000) {

- lastCycle = millis();

- lcd.clear();

- switch (displayPage) {

- case 0: lcd.print("Temp: " + String(temp) + "C"); break;

- case 1: lcd.print("Hum: " + String(hum) + "%"); break;

- case 2: lcd.print("Light: " + String(avgLdr)); break;

- }

- displayPage = (displayPage + 1) % 3;

- Serial.print("T="); Serial.print(temp);

- Serial.print(" H="); Serial.print(hum);

- Serial.print(" L="); Serial.println(avgLdr);

- // BONUS: MQTT publish (requires Wi-Fi + PubSubClient library)

- // mqttClient.publish("greenhouse/temp", String(temp).c_str());

- }

}

Design notes: Hysteresis bands prevent relay "chattering" right at the setpoint boundary, extending relay lifespan and reducing power-supply stress — a robustness detail evaluators look for under "System Architecture."

### Q46. River Water Level Alert (Flood Detection)

Approach: Measure distance to water surface with HC-SR04 (smaller distance = higher water), apply a 10-sample rolling average to filter ultrasonic noise, and classify into NORMAL/WARNING/CRITICAL bands with distinct LED/buzzer patterns, broadcasting the alert level as a simple string.

cpp

- #define TRIG_PIN 5

- #define ECHO_PIN 18

- #define LED_GREEN 25

- #define LED_YELLOW 26

- #define LED_RED 27

- #define BUZZER_PIN 14

- const int SAMPLES = 10;

- float readings[SAMPLES];

- int idx = 0;

- float total = 0;

- float readDistanceCM() {

- digitalWrite(TRIG_PIN, LOW); delayMicroseconds(2);

- digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10);

- digitalWrite(TRIG_PIN, LOW);

- long duration = pulseIn(ECHO_PIN, HIGH, 30000); // 30ms timeout

- return duration * 0.0343 / 2.0;

- }

- float updateRollingAverage(float newVal) {

- total -= readings[idx];

- readings[idx] = newVal;

- total += newVal;

- idx = (idx + 1) % SAMPLES;

- return total / SAMPLES;

- }

- void setup() {

- Serial.begin(115200);

- pinMode(TRIG_PIN, OUTPUT);

- pinMode(ECHO_PIN, INPUT);

- pinMode(LED_GREEN, OUTPUT);

- pinMode(LED_YELLOW, OUTPUT);

- pinMode(LED_RED, OUTPUT);

- pinMode(BUZZER_PIN, OUTPUT);

- for (int i = 0; i < SAMPLES; i++) readings[i] = 100; // assume normal clearance at boot

- total = 100 * SAMPLES;

- }

- void loop() {

- float raw = readDistanceCM();

- float avgClearance = updateRollingAverage(raw);

- String level;

- digitalWrite(LED_GREEN, LOW);

- digitalWrite(LED_YELLOW, LOW);

- digitalWrite(LED_RED, LOW);

- noTone(BUZZER_PIN);

- if (avgClearance > 30) {

- level = "NORMAL";

- digitalWrite(LED_GREEN, HIGH);

- } else if (avgClearance > 15) {

- level = "WARNING";

- digitalWrite(LED_YELLOW, HIGH);

- tone(BUZZER_PIN, 1000, 200);

- } else {

- level = "CRITICAL";

- digitalWrite(LED_RED, HIGH);

- tone(BUZZER_PIN, 2500, 400);

- }

- Serial.print("Clearance="); Serial.print(avgClearance);

- Serial.print("cm Level="); Serial.println(level);

- // Wi-Fi/BT broadcast: e.g., Serial2.println(level); or MQTT publish.

- // A web dashboard using Chart.js polls a small backend/WebSocket bridge

- // fed by this MQTT topic to plot avgClearance as a live line graph.

- delay(500);

}

Design notes: The rolling average smooths out noisy reflections common with HC-SR04 near a rippling water surface.

### Q47. Heart Rate & SpO2 Monitor (Simulation)

Approach: Since this is explicitly a learning simulation (not a medical device), we simulate physiological-like data using an LDR (photoplethysmography proxy) or, if hardware is available, read directly from a MAX30102 over I2C. A 5-reading rolling average smooths noise, and an LED flags abnormal readings.

cpp

- // Simulation version using LDR as a PPG proxy (documented clearly as non-medical)

- #define LDR_PIN 34

- #define ALERT_LED 25

- const int SAMPLES = 5;

- int hrBuffer[SAMPLES];

- int spo2Buffer[SAMPLES];

- int bufIndex = 0;

- unsigned long lastUpdate = 0;

- const unsigned long UPDATE_INTERVAL = 2000;

- void setup() {

- Serial.begin(115200);

- pinMode(ALERT_LED, OUTPUT);

- for (int i = 0; i < SAMPLES; i++) { hrBuffer[i] = 75; spo2Buffer[i] = 97; }

- }

- int rollingAverage(int arr[]) {

- long sum = 0;

- for (int i = 0; i < SAMPLES; i++) sum += arr[i];

- return sum / SAMPLES;

- }

- // Simulated read — replace with real MAX30102 library calls (e.g., SparkFun MAX3010x)

- int simulateHR() {

- int ldrVal = analogRead(LDR_PIN);

- return map(ldrVal, 0, 4095, 55, 130);

- }

- int simulateSpO2() {

- int ldrVal = analogRead(LDR_PIN);

- return map(ldrVal, 0, 4095, 90, 100);

- }

- void loop() {

- if (millis() - lastUpdate >= UPDATE_INTERVAL) {

- lastUpdate = millis();

- hrBuffer[bufIndex] = simulateHR();

- spo2Buffer[bufIndex] = simulateSpO2();

- bufIndex = (bufIndex + 1) % SAMPLES;

- int avgHR = rollingAverage(hrBuffer);

- int avgSpO2 = rollingAverage(spo2Buffer);

- bool abnormal = (avgHR < 50 || avgHR > 120 || avgSpO2 < 94);

- digitalWrite(ALERT_LED, abnormal ? HIGH : LOW);

- Serial.print("HR="); Serial.print(avgHR);

- Serial.print("bpm SpO2="); Serial.print(avgSpO2);

- Serial.print("% Status="); Serial.println(abnormal ? "ABNORMAL" : "NORMAL");

- // Append to /data/health_log.csv (documented in README; on-device this would

- // write to an SD card module, or be captured to a host file via Serial logging)

- }

}

Design notes: This remains explicitly a sensor-interfacing learning exercise — the README should clearly disclaim that readings are simulated/illustrative and not clinically valid, per the problem statement.

## Open-Ended Design Questions

### Q48. Design Challenge: IoT for Jammu Smart City (500–700 words)

Selected systems: Street Lighting, Traffic Flow, Waste Bin Fill Levels

1. Smart Street Lighting Jammu's street lighting currently runs on fixed timers, wasting energy on empty roads late at night and under-lighting during heavy fog or early monsoon darkness. An IoT-based adaptive lighting system would use an LDR (ambient light sensor) paired with a PIR motion sensor at each pole to detect approaching pedestrians or vehicles and brighten LEDs only when needed, dimming to a low standby level otherwise.

- Sensors: LDR (ambient light), PIR (motion presence).

- Microcontroller/connectivity: ESP32 (Wi-Fi) at each pole cluster, or a LoRa module where poles are far from Wi-Fi coverage, reporting to a central gateway.

- Data flow: LDR/PIR to ESP32 (local dimming decision) to LoRa/Wi-Fi to Cloud (fault/energy dashboard) to Municipal Dashboard (map view of lit/unlit/faulty poles).

- Estimated cost per node: ESP32 (Rs.450) + LDR (Rs.20) + PIR (Rs.80) + driver circuit (Rs.150) is approximately Rs.700–900 per pole, excluding the LED fixture itself.

2. Traffic Flow Monitoring Jammu's key junctions suffer congestion with no adaptive signal timing. An ultrasonic/IR vehicle-counting array at each approach lane can estimate queue length and vehicle throughput, feeding a signal-timing optimization service.

- Sensors: IR break-beam or ultrasonic (HC-SR04) sensors per lane, or a low-cost camera module with on-device object counting for busier junctions.

- Microcontroller/connectivity: ESP32-CAM (where vision is used) or ESP32 with ultrasonic sensors, connected via 4G/LTE for junctions outside stable Wi-Fi range, or Wi-Fi where municipal networks exist.

- Data flow: Sensor array to ESP32 (vehicle count/queue estimate) to MQTT over 4G/Wi-Fi to Cloud traffic-analytics service to Traffic Control Dashboard (adjusts signal timing recommendations) and public traffic app.

- Estimated cost per node: ESP32-CAM (Rs.1,200) or ESP32 + 4x HC-SR04 (Rs.450+Rs.400) + 4G module (Rs.900) is approximately Rs.1,500–2,500 per junction lane set.

3. Waste Bin Fill-Level Monitoring Municipal waste collection in Jammu often runs on a fixed schedule regardless of actual bin fill level, leading to overflowing bins in high-traffic areas and wasted collection trips to near-empty bins elsewhere. Mounting an ultrasonic sensor inside each public bin lid to measure fill height solves this.

- Sensors: HC-SR04 ultrasonic sensor (mounted at the lid, facing down into the bin).

- Microcontroller/connectivity: ESP8266 (low-cost, low-power) with deep-sleep between readings (e.g., hourly), reporting over Wi-Fi where available or LoRa in outlying areas, since bins don't need constant connectivity.

- Data flow: HC-SR04 to ESP8266 (fill % calculation, deep-sleep cycle) to LoRa/Wi-Fi to Cloud (MQTT ingestion) to Waste Management Dashboard (route optimization for collection trucks, prioritizing bins over 80% full).

- Estimated cost per node: ESP8266 (Rs.250) + HC-SR04 (Rs.80) + battery/solar trickle charger (Rs.300) is approximately Rs.600–700 per bin.

Common architecture: All three systems funnel into a shared MQTT broker hosted on a municipal cloud instance, which routes topic-specific data (streetlight/#, traffic/#, wastebin/#) to a unified city-operations dashboard (e.g., Grafana or a custom Node-RED interface). This shared backbone reduces integration cost, lets the city add new sensor categories later without redesigning the pipeline, and keeps per-node hardware cost low (Rs.600–2,500 depending on system) — a realistic, incrementally deployable path to a Jammu smart city pilot.

(This section is written to be saved as /final_project/smart_city_proposal.md in the GitHub repository, per the assignment instructions. Code for each of the three sub-systems should be written and committed separately, as instructed.)

### Q49. Security Vulnerability Analysis

Scenario: An ESP8266 connects to public Wi-Fi with hardcoded SSID/password, uses plain HTTP, publishes to a public unauthenticated MQTT broker, and never validates incoming commands.

Five vulnerabilities and their risks:

- Hardcoded SSID/password in source code. Anyone with access to the code (e.g., a public GitHub repo) obtains the Wi-Fi credentials directly, potentially compromising the entire network the device sits on, not just the device itself.

- Unencrypted HTTP instead of HTTPS. All data — sensor readings, any tokens, command payloads — travels in plaintext. An attacker on the same public Wi-Fi network can trivially sniff this traffic (e.g., with Wireshark) or perform a man-in-the-middle attack to alter data in transit.

- Publishing to a public MQTT broker with no authentication. Anyone who knows (or guesses) the broker address and topic name can subscribe to the device's data feed, read sensitive sensor data, or — worse — publish spoofed messages on the same topic, corrupting or hijacking the data stream.

- No validation of incoming commands. If the device blindly executes any command it receives on its subscribed topic (e.g., "unlock door", "activate relay"), an attacker who reaches that public broker can issue arbitrary commands to the device with zero authorization checks — a direct path to unauthorized physical actuation.

- No firmware/update integrity checking (implied by "never validates"). If the device also accepts firmware updates without checking a signature or checksum, an attacker could push malicious firmware disguised as a legitimate update, achieving full device takeover.

Corrected code for two vulnerabilities:

(1) Moving credentials out of source code, into a git-ignored config file:

cpp

- // config.h  (added to .gitignore — never committed)

- #define WIFI_SSID "your_network_name"

- #define WIFI_PASS "your_secure_password"

- #define MQTT_USER "device_client_01"

- #define MQTT_PASS "strong_broker_password"

- // main.ino

- #include "config.h"

- #include <ESP8266WiFi.h>

- void setup() {

- WiFi.begin(WIFI_SSID, WIFI_PASS); // credentials no longer visible in shared code

}

(3) Requiring authentication on MQTT connect, and (4) validating commands before acting on them:

cpp

- #include <PubSubClient.h>

- WiFiClientSecure espClient; // TLS-capable client instead of plain WiFiClient

- PubSubClient mqttClient(espClient);

- void connectMQTT() {

- // Authenticated, encrypted connection instead of anonymous plaintext

- while (!mqttClient.connected()) {

- mqttClient.connect("esp8266_client_01", MQTT_USER, MQTT_PASS);

- }

- }

- void callback(char* topic, byte* payload, unsigned int length) {

- String message;

- for (unsigned int i = 0; i < length; i++) message += (char)payload[i];

- // Validate command against an allow-list before ever acting on it

- const String allowedCommands[] = {"STATUS", "PING", "RELAY_ON", "RELAY_OFF"};

- bool valid = false;

- for (String cmd : allowedCommands) {

- if (message == cmd) { valid = true; break; }

- }

- if (valid) {

- executeCommand(message);

- } else {

- Serial.println("REJECTED unrecognized command: " + message);

- }

}

Together, these fixes address credential exposure, transport encryption, broker authentication, and command validation — the four most exploitable issues in the original design.

### Q50. Reflection & Learning Portfolio (min. 400 words)

Over this assignment, I moved from single-sensor exercises to full end-to-end IoT systems — reading data from sensors like the DHT11, MQ-2, HC-SR04, and a PIR motion detector; processing that data with debouncing, rolling averages, and hysteresis logic; and acting on it through relays, servos, buzzers, and LCD displays. I also worked through the networking side: comparing MQTT and HTTP, understanding QoS guarantees, and seeing firsthand why unauthenticated public brokers and hardcoded credentials are such common real-world vulnerabilities rather than just textbook warnings.

The biggest learning shift for me was realizing that "getting the sensor to give a number" is the easy part — the real engineering is in the decision logic around that number: what threshold counts as dangerous smoke, how do you avoid false triggers from noisy ultrasonic echoes, how do you keep a keypad responsive during a 30-second countdown without blocking the whole program with delay(). Working through the water-level and greenhouse projects in particular taught me to think in terms of rolling averages and hysteresis bands rather than raw instantaneous readings — small design choices that separate a fragile demo from something that could survive real deployment conditions.

The security-focused questions were an unexpected highlight. Analyzing a vulnerable IoT snippet and having to name specific exploitable weaknesses — not just "it's insecure," but exactly why an unauthenticated public MQTT broker lets an attacker inject spoofed commands — made the abstract idea of "IoT security" concrete. I now think about every project, even class demos, in terms of: where do secrets live, is the transport encrypted, and does the device validate what it's told to do.

I can already see IoT skills applying directly to my field of study. Environmental and agricultural sensing (soil moisture, greenhouse control) connects to sustainability-focused problems I care about, and the security analysis work is directly transferable to any embedded or backend systems role, since the same "never trust unvalidated input" principle applies far beyond IoT.

For my next project, I'd like to build a campus energy-monitoring system: clamp-on current sensors (non-invasive, around classroom AC/lighting circuits) reporting to an ESP32 gateway over MQTT, aggregating usage per building block, and surfacing a dashboard that highlights which rooms are running HVAC or lights when unoccupied — cross-referenced with a PIR-based occupancy sensor, similar to the security-trigger project here. The interesting technical challenge would be doing this at scale, dozens of sensor nodes per building, while keeping the MQTT topic structure organized and the broker properly authenticated, learning directly from the Q49 exercise. Longer term, I'd want to add a simple anomaly-detection model on the aggregated data, extending the "rolling average" idea from the health-monitor project into a larger scale predictive pipeline that flags abnormal energy usage patterns automatically.